Custom 8-bit RISC CPU

Digital Circuit Design Reference

Version 0.3  ·  Multi-cycle Branching + Memory Support

Primary Reference: cpu_digital/ (.dig circuit files)

Architecture Summary

| Datapath width | 8-bit |
| --- | --- |
| Address / PC bus | 16-bit |
| Instruction width | 16-bit fixed |
| Memory architecture | Harvard (IMEM ROM + DMEM RAM) |
| Execution model | Multi-cycle FSM (4 states) |
| ISA style | RISC load/store |
| General-purpose registers | 8 × 8-bit (R0–R7, R0 hardwired 0) |
| Implemented opcodes | 14 (0x0–0xD) |
| Tool — circuit schematic | Digital (Hneemann) .dig files |
| Tool — simulation | Icarus Verilog + GTKWave |
| Assembler | Python two-pass (assembler.py) |
| Status | Active development — v0.3 |

# 1.  System Architecture

The CPU is implemented as a Harvard multi-cycle RISC processor. The primary design artefacts are the Digital circuit files (cpu_digital/*.dig). The Verilog source files in src/ are a faithful RTL translation of those schematics and are used for simulation only.

## 1.1  Datapath Overview

Data flows through five major blocks every instruction cycle:

- Fetch Unit — program counter (PC), instruction memory (ROM), instruction register (IR)

- Decoder — combinational 16-bit splitter extracting opcode, rd, rs, imm6

- Control Unit — 2-bit FSM counter driving all control signals purely combinationally

- Execution core — register file, ALU, data memory

- Writeback mux — selects between ALU result and memory read data

## 1.2  Instruction Encoding

All instructions are exactly 16 bits wide with a fixed four-field format:

Bit positions:  [15:12]    [11:9]    [8:6]    [5:0]

Field name:      opcode      rd        rs      imm6

Width:           4 bits    3 bits   3 bits   6 bits

opcode — selects the operation (14 defined, 2 reserved).

rd — destination register (also first ALU source for register-register ops).

rs — second source register; unused for ADDI, LOAD, STORE, JMP.

imm6 — 6-bit immediate field. Sign-extended to 8 bits for ADDI ALU input; sign-extended to 16 bits for branch offsets; zero-extended to 8 bits for LOAD/STORE addresses; zero-extended to 16 bits for JMP absolute target.

## 1.3  ISA — Opcode Table

The Digital comparator constants in control_unit.dig match the opcode values below exactly.

| Opcode (bin) | Hex | Mnemonic | Operation | Flags updated |
| --- | --- | --- | --- | --- |
| 0000 | 0x0 | NOP | No operation | — |
| 0001 | 0x1 | ADD | Rd ← Rd + Rs | Z N C V |
| 0010 | 0x2 | SUB | Rd ← Rd − Rs | Z N C V |
| 0011 | 0x3 | AND | Rd ← Rd & Rs | Z N |
| 0100 | 0x4 | OR | Rd ← Rd ¦ Rs | Z N |
| 0101 | 0x5 | XOR | Rd ← Rd ^ Rs | Z N |
| 0110 | 0x6 | LOAD | Rd ← MEM[zero_ext(imm6)] | — |
| 0111 | 0x7 | STORE | MEM[zero_ext(imm6)] ← Rd | — |
| 1000 | 0x8 | MOV | Rd ← Rs (imm6 ignored) | Z N |
| 1001 | 0x9 | ADDI | Rd ← Rd + sign_ext(imm6) | Z N C V |
| 1010 | 0xA | JMP | PC ← zero_ext(imm6) (absolute) | — |
| 1011 | 0xB | BEQ | if Z=1: PC ← PC+1 + sign_ext(imm6) | — |
| 1100 | 0xC | BNE | if Z=0: PC ← PC+1 + sign_ext(imm6) | — |
| 1101 | 0xD | BLT | if N⊕V=1: PC ← PC+1 + sign_ext(imm6) | — |
| 1110 | 0xE | (rsvd) | Reserved | — |
| 1111 | 0xF | (rsvd) | Reserved | — |

AND, OR, XOR, MOV: carry (C) and overflow (V) are set to zero by the ALU MUX topology — the adder/subtractor outputs are not routed to those flag paths.

## 1.4  ALU Flags

| Flag | Bit | Circuit element | Condition |
| --- | --- | --- | --- |
| Z — Zero | 0 | Comparator(8-bit) result == 0x00 | result == 0 |
| C — Carry | 1 | 2-input MUX: sel=(opcode==SUB) | 9th bit of Add or Sub unit output |
| N — Negative | 2 | Splitter → bit[7] of result bus | result[7] = 1 |
| V — Overflow | 3 | XNOR/XOR/AND gate tree + 1-bit MUX | Signed overflow; see §4.7 |

# 2.  Multi-Cycle FSM

## 2.1  State Encoding

The FSM is implemented in control_unit.dig as a 2-bit counter register. On every rising clock edge the counter increments by 1 modulo 4 using a 2-bit Add element and a Mux that injects 2'b00 on reset.

| State | Encoding | Digital element | What happens |
| --- | --- | --- | --- |
| FETCH | 2'b00 | Comparator == Const(0) | pc_enable=1 increments PC; ir_load=1 latches instruction into IR |
| DECODE | 2'b01 | Comparator == Const(1) | Combinational decode only; no registers change |
| EXECUTE | 2'b10 | Comparator == Const(2) | ALU computes; mem_read asserted for LOAD |
| WRITEBACK | 2'b11 | Comparator == Const(3) | Register write, memory write, or PC load committed |

## 2.2  State Transition

Transitions are unconditional — every instruction takes exactly 4 cycles. There are no conditional state jumps in v0.3.

FETCH (00)  →  DECODE (01)  →  EXECUTE (10)  →  WRITEBACK (11)  →  FETCH (00)

+1              +1               +1                  +1 (wraps)

## 2.3  Control Signal Truth Table

All outputs from control_unit.dig are purely combinational (AND/OR trees off state comparators and opcode comparators). There are no latch registers on alu_op or alu_src.

| Instruction | FETCH | DECODE | EXECUTE | WRITEBACK |
| --- | --- | --- | --- | --- |
| NOP | pc_en ir_load | — | — | — |
| ADD / SUB / AND / OR / XOR | pc_en ir_load | — | alu_op=opcode, alu_src=0 | reg_write |
| ADDI | pc_en ir_load | — | alu_op=0x9, alu_src=1 | reg_write |
| MOV | pc_en ir_load | — | alu_op=0x8, alu_src=0 | reg_write |
| LOAD | pc_en ir_load | — | mem_read | mem_read reg_write |
| STORE | pc_en ir_load | — | — | mem_write |
| JMP | pc_en ir_load | — | — | pc_load (unconditional) |
| BEQ | pc_en ir_load | — | alu_op=0x2 (SUB) | pc_load if Z=1 |
| BNE | pc_en ir_load | — | alu_op=0x2 (SUB) | pc_load if Z=0 |
| BLT | pc_en ir_load | — | alu_op=0x2 (SUB) | pc_load if N⊕V=1 |

alu_op and alu_src are active in both EXECUTE and WRITEBACK because the .dig circuit drives them directly from the current opcode with no pipeline register. The ALU recomputes continuously in both cycles.

## 2.4  ALU_OP Selection Logic in control_unit.dig

The alu_op output is built by a chain of five 2-input 4-bit Multiplexers. Reading right-to-left in the schematic (most-recently-overriding wins):

- Default (all states, all opcodes): 0x0 from Ground(0x00, 4-bit)

- EXECUTE or WRITEBACK, arithmetic/MOV/ADDI: opcode value passed directly — the opcode IS the alu_op (0x1–0x5, 0x8, 0x9)

- EXECUTE or WRITEBACK, BEQ/BNE/BLT: forced to 0x2 (SUB) for comparison

- LOAD, STORE, JMP: 0x0 (ALU result not used)

This means the ALU runs idle (opcode=0) during FETCH and DECODE for every instruction, and runs the correct operation in both EXECUTE and WRITEBACK.

# 3.  Component-by-Component Circuit Walkthrough

Each subsection describes a single .dig file. Digital element names are used exactly as they appear in the XML. Wire coordinates are abbreviated to direction (→ = signal flow).

## 3.1  decoder.dig — Instruction Decoder

The simplest component: a single Digital Splitter element with no clock.

| Element | Type | Configuration | Purpose |
| --- | --- | --- | --- |
| instruction_in | In | 16-bit | Raw instruction word from IR |
| Splitter_0 | Splitter | Input: 16 Output: 6,3,3,4 (splitterSpreading=4) | Slices the 16-bit word into four fields |
| imm6 | Out | 6-bit | instruction_in[5:0] |
| rs | Out | 3-bit | instruction_in[8:6] |
| rd | Out | 3-bit | instruction_in[11:9] |
| opcode | Out | 4-bit | instruction_in[15:12] |

The Digital Splitter in LSB-first mode outputs the lowest-numbered bits first. The 6,3,3,4 split starting from bit 0 gives imm6=[5:0], rs=[8:6], rd=[11:9], opcode=[15:12]. This maps exactly to the ISA encoding.

## 3.2  program_counter.dig — Program Counter

A 16-bit counter with three priority-selected inputs built from three Multiplexers, an Adder, and one Register.

| Element | Configuration | Signal flow |
| --- | --- | --- |
| Add(16-bit) | A=pc_out, B=Const(1), Cin=Const(0) | Produces pc_out+1 combinationally |
| MUX1 (pc_enable) | sel=pc_enable, in[0]=pc_out, in[1]=pc_out+1 | Holds or increments |
| MUX2 (pc_load) | sel=pc_load, in[0]=MUX1_out, in[1]=pc_in | Selects branch/JMP target |
| MUX3 (reset) | sel=reset, in[0]=MUX2_out, in[1]=Ground(0x0000) | Forces 0 on reset |
| Register(16-bit) | D=MUX3_out, en=Const(1), clk=clk | Stores PC; always-enabled |
| pc_out | Register Q | Fed back to Adder and to instruction_memory.address |

Priority (innermost MUX wins last, i.e. highest priority):

- reset > pc_load > pc_enable > hold

The Register enable pin is hardwired to Const(1), meaning the register clocks every cycle. The MUX3 output of 0x0000 on reset is what actually resets the counter — not a synchronous reset input on the Register itself.

## 3.3  instruction_register.dig — Instruction Register

A 16-bit register with a conditional load gate implemented using a MUX and an OR gate.

| Element | Configuration | Notes |
| --- | --- | --- |
| MUX(16-bit) | in[0]=Ground(0x0000), in[1]=instruction_in, sel=OR_out | Passes 0 or instruction_in |
| OR gate | in[0]=reset, in[1]=ir_load | Gate enable — register only clocks when one of these is high |
| Register(16-bit) | D=MUX_out, en=OR_out, clk=clk | Latches on posedge when enabled |
| Ground(0x0000,16-bit) | Constant 0 → MUX in[0] | Clears IR on reset |

Behaviour summary:

- reset=1, ir_load=0: OR=1, MUX selects 0x0000, register latches 0x0000

- reset=0, ir_load=1: OR=1, MUX selects instruction_in, register latches instruction

- reset=0, ir_load=0: OR=0, register clock-enable is low — instruction_out holds previous value

The register is enabled only in FETCH (ir_load=1) or during active reset. This is more power-efficient than a continuously clocking register with a conditional write.

## 3.4  instruction_memory.dig — Instruction ROM

| Element | Configuration | Notes |
| --- | --- | --- |
| ROM | AddrBits=8, Bits=16, 12 data entries | Asynchronous read — no clock pin |
| Const(1) | 1-bit → ROM enable | ROM is always enabled |
| address | In, 8-bit | Driven by pc_out[7:0] from cpu_core_test.dig |
| instruction | Out, 16-bit | Combinationally valid whenever address changes |

ROM contents (from instruction_memory.dig data string, corrected to match assembler output):

| Address | Hex word | Disassembly |
| --- | --- | --- |
| 0x00 | 0x9205 | ADDI R1, #5 |
| 0x01 | 0x9405 | ADDI R2, #5 |
| 0x02 | 0xB281 | BEQ R1, R2, +1 (branch to 0x04, skip 0x03) |
| 0x03 | 0x9601 | ADDI R3, #1 (skipped) |
| 0x04 | 0x9809 | ADDI R4, #9 |
| 0x05 | 0x780A | STORE R4, #10 |
| 0x06 | 0x6A0A | LOAD R5, #10 |
| 0x07 | 0x9C05 | ADDI R6, #5 (loop counter init) |
| 0x08 | 0x9C3F | ADDI R6, #-1 (LOOP top) |
| 0x09 | 0xDC01 | BLT R6, R0, +1 (exit loop) |
| 0x0A | 0xB03D | BEQ R0, R0, -3 (unconditional back to 0x08) |
| 0x0B | 0x0000 | NOP (END sentinel) |
| 0x0C–0xFF | 0x0000 | NOP (all remaining slots zeroed) |

The .dig file in the repository stores 0xA00B (JMP #11) at address 11 — this would cause an infinite loop. The correct value is 0x0000 (NOP) which matches the assembler output in build/example.hex. Update the ROM data string in instruction_memory.dig accordingly.

## 3.5  data_memory.dig — Data RAM

| Element | Configuration | Notes |
| --- | --- | --- |
| RAMDualPort | AddrBits=8, Bits=8 | 256 × 8-bit synchronous dual-port RAM |
| address (In, 8-bit) | → both Port A and Port B address | Same address bus serves both read and write |
| write_data (In, 8-bit) | → Port A data input | Source register value (reg_data1 from cpu_core) |
| mem_write (In, 1-bit) | → Port A write-enable | Asserted in WRITEBACK for STORE only |
| mem_read (In, 1-bit) | → Port B read-enable | Asserted in EXECUTE and WRITEBACK for LOAD |
| clk (In, 1-bit) | → both port clocks | Both ports synchronous on posedge |
| read_data (Out, 8-bit) | ← Port B data output | Valid one cycle after mem_read asserted |

The two-cycle LOAD sequence is a direct consequence of the RAMDualPort registered output:

- EXECUTE: mem_read=1 asserted; RAMDualPort captures address on this posedge

- WRITEBACK: mem_read=1 still held; read_data is now valid; reg_write=1 commits data to register file

The address to the RAM is {2'b00, imm6} — zero-extending the 6-bit immediate to 8 bits. This restricts data memory access to addresses 0–63 in v0.3.

## 3.6  reg_file.dig — Register File

Eight 8-bit registers (R0–R7) with one write port and two independent read ports. R0 is permanently hardwired to zero.

### Write path

| Element | Configuration | Role |
| --- | --- | --- |
| Decoder(Selector Bits=3) | in=write_reg[2:0], out=8 one-hot lines | Decodes write address to one-hot |
| AND gate ×7 (R1–R7) | in[0]=decoder_out[n], in[1]=reg_write | Gate write-enable per register |
| Register(8-bit) ×8 | D=write_data, en=AND[n]_out, clk=clk | Stores register value |
| R0: Const(0) on D + AND disabled | D=Const(0), en never asserted | R0 hardwired zero — never written |

### Read paths (two identical structures)

| Element | Configuration | Role |
| --- | --- | --- |
| MUX1 (read_data1) | Bits=8, Selector Bits=3, sel=read_reg1, in[0]=Const(0x00), in[1..7]=R1..R7.Q | Async read port 1 |
| MUX2 (read_data2) | Bits=8, Selector Bits=3, sel=read_reg2, in[0]=Const(0x00), in[1..7]=R1..R7.Q | Async read port 2 |

Read is asynchronous — no clock. read_data1 and read_data2 are valid combinationally as soon as read_reg1/read_reg2 are stable. This allows the ALU inputs to be ready during EXECUTE without an extra cycle.

In cpu_core_test.dig the reg_file ports are connected as: read_reg1=rd, read_reg2=rs, write_reg=rd. Both source and destination use the rd field — the rs field only reaches read_reg2 for the second ALU operand.

## 3.7  alu.dig — Arithmetic-Logic Unit

All five operation units run in parallel continuously. A 16-input 8-bit Multiplexer selects the correct result using alu_op as selector.

### Operation units (always active)

| Digital element | Bits | Inputs | Output routed to MUX input |
| --- | --- | --- | --- |
| Add(8-bit) | 8 | a, b | MUX[0x1] and MUX[0x9] (ADD and ADDI share adder) |
| Sub(8-bit) | 8 | a, b | MUX[0x2] |
| And(8-bit) | 8 | a, b | MUX[0x3] |
| Or(8-bit) | 8 | a, b | MUX[0x4] |
| XOr(8-bit) | 8 | a, b | MUX[0x5] |
| Ground(0x00) | 8 | — | MUX[0x0], MUX[0x6], MUX[0x7], MUX[0xA..0xF] |
| b bus (wire) | 8 | b input directly | MUX[0x8] (MOV: pass b through) |

### Result MUX

Multiplexer(Bits=8, Selector Bits=4) — 16 inputs, selector=alu_op. The MUX output is the result output of the ALU.

### Flag generation

| Flag | Digital circuit in alu.dig | Formula |
| --- | --- | --- |
| zero | Comparator(8-bit): result == 0x00 | result == 8'h00 |
| negative | Splitter(8→1,1,1,1,1,1,1,1) bit[7] | result[7] |
| carry | 2-input MUX: sel=Comparator(alu_op==2) in[0]=Add carry bit [8] in[1]=Sub borrow bit [8] | alu_op==SUB ? sub_full[8] : add_full[8] |
| overflow | Three Splitters (a[7], b[7], result[7]) XNOR(a[7],b[7]) → signs_equal XOR(a[7],b[7]) → signs_differ XOR(result[7],a[7]) → sign_changed AND(signs_equal,sign_changed) → ov_add AND(signs_differ,sign_changed) → ov_sub 1-bit MUX: sel=(alu_op==2) → ov_sub : ov_add | ADD: (a[7]==b[7]) & (result[7]!=a[7]) SUB: (a[7]!=b[7]) & (result[7]!=a[7]) |

## 3.8  control_unit.dig — Control Unit

The most complex .dig file. Contains the FSM counter, 13 opcode comparators, 4 state comparators, and all the AND/OR trees that generate control signals.

### FSM counter circuit

| Element | Configuration | Role |
| --- | --- | --- |
| Register(2-bit) | D=MUX_out, en=Const(1), clk=clk | Holds current state; always-enabled |
| Add(2-bit) | A=state, B=Const(1), Cin=Const(0) | state + 1 combinationally |
| Const(1,2-bit) | 1 on Add B input | The increment value |
| Const(0,2-bit) | → Mux reset input | Reset target state |
| Mux(2-bit) | sel=reset, in[0]=Add_out, in[1]=Const(0,2-bit) | Reset injection |

### State decode (four Comparators, 2-bit)

is_FETCH     = (state == 2'b00)    Const(0)

is_DECODE    = (state == 2'b01)    Const(1)

is_EXECUTE   = (state == 2'b10)    Const(2)

is_WRITEBACK = (state == 2'b11)    Const(3)

### Opcode decode (13 Comparators, 4-bit)

Each comparator has one input from the opcode bus and one from a constant:

| Signal | Const value | Signal | Const value |
| --- | --- | --- | --- |
| is_ADD | Const(1) | is_STORE | Const(7) |
| is_SUB | Const(2) | is_MOV | Const(8) |
| is_AND | Const(3) | is_ADDI | Const(9) |
| is_OR | Const(4) | is_JMP | Const(10) |
| is_XOR | Const(5) | is_BEQ | Const(11) |
| is_LOAD | Const(6) | is_BNE | Const(12) |
| — | — | is_BLT | Const(13) |

### Output signal generation (AND/OR trees — no latches)

| Output | Digital gates | Logic |
| --- | --- | --- |
| ir_load | AND(is_FETCH, Const(1)) | is_FETCH |
| pc_enable | AND(is_FETCH, Const(1)) | is_FETCH |
| mem_write | AND(is_WRITEBACK, is_STORE) | is_WRITEBACK & is_STORE |
| mem_read | AND(is_EXECUTE,is_LOAD) + AND(is_WRITEBACK,is_LOAD) → OR | (is_EXECUTE¦is_WRITEBACK) & is_LOAD |
| reg_write | 8-input OR of AND(is_WB, is_X) for X∈{ADD,SUB,AND,OR,XOR,MOV,ADDI,LOAD} | is_WRITEBACK & (arith¦MOV¦ADDI¦LOAD) |
| alu_src | AND(is_EXECUTE¦is_WRITEBACK, is_ADDI) | (is_EXECUTE¦is_WRITEBACK) & is_ADDI |
| alu_op | 5-MUX chain (see §2.4) | Combinational off opcode+state |
| pc_load | 4-input OR: AND(is_WB,is_JMP) ¦ AND(is_WB,is_BEQ,zero_flag) ¦ AND(is_WB,is_BNE,NOT(zero_flag)) ¦ AND(is_WB,is_BLT,XOR(neg,ov)) | Conditional on branch flags |

There are NO pipeline registers (latches) between EXECUTE and WRITEBACK for alu_op or alu_src in the .dig circuit. The Verilog file src/control_unit.v was updated to match this: both signals are purely combinational.

## 3.9  cpu_core_test.dig — Top-Level Integration

This is the system integration schematic. It instantiates all nine sub-components and wires them together. Notable top-level elements not present in sub-components:

### Splitter — PC address extraction

A Splitter(Input: 16, Output: 8,8) sits between program_counter.pc_out and instruction_memory.address. It extracts the lower 8 bits [7:0] as the ROM address, discarding the upper 8 bits. This limits the instruction memory to 256 words (sufficient for v0.3).

### BitExtender — imm6 sign extension for ADDI (6→8)

BitExtender(inputBits=6, outputBits=8, rotation=3 [downward input], signed) expands imm6 from 6 bits to 8 bits using sign extension. This is the B input to the ALU for ADDI instructions. The sign bit (imm6[5]) is replicated into bits [7:6], so ADDI R6, #-1 (imm6=111111) produces 11111111 = 0xFF = -1 in the ALU.

### BitExtender — imm6 sign extension for branches (6→16)

A second BitExtender(inputBits=6, outputBits=16, rotation=1 [right-input], signed) expands imm6 to 16 bits for the branch offset computation. The 16-bit Add element then computes: branch_target = pc_out + sign_ext_16(imm6). Since pc_out is already PC+1 (incremented during FETCH), the branch target is relative to the instruction after the branch.

### Splitter — JMP absolute target construction (6,10→16)

A Splitter(Input Splitting: 6,10, Output Splitting: 16, rotation=1) combines imm6[5:0] as the low 6 bits with Const(0, 10-bit) as the upper 10 bits to form the 16-bit JMP absolute address {10'b0, imm6}. This limits JMP to addresses 0–63.

### Comparator + MUX — JMP vs branch target select

Comparator(4-bit) checks opcode == Const(10) [JMP]. The 1-bit result drives a Multiplexer(16-bit): sel=is_JMP, in[0]=branch_target (PC-relative), in[1]=jmp_target (absolute). The output pc_next feeds program_counter.pc_in.

### MUX — ALU B input select

Multiplexer(8-bit): sel=alu_src, in[0]=reg_data2 (register operand), in[1]=sign_ext_8 (from 6→8 BitExtender). alu_src is only high for ADDI.

### MUX — Writeback data select

Multiplexer(8-bit): sel=mem_read, in[0]=alu_result, in[1]=mem_data. mem_read is high in WRITEBACK for LOAD, so mem_data routes to the register file write port.

### Probe elements

Four Probe elements in cpu_core_test.dig attach to key signals for in-simulator debugging: pc_out, alu_op, instruction word, and flags bus. They have no effect on circuit logic.

### Clock element

Clock(Frequency=10, runRealTime=true) — 10 Hz real-time clock for interactive demonstration in the Digital simulator. The Icarus Verilog testbench uses a 10 ns period clock instead (matching clock period does not affect correctness).

# 4.  Branching and PC Arithmetic

## 4.1  Branch Timing (4-cycle detail)

| Cycle | State | PC register (pc_out) | Key action |
| --- | --- | --- | --- |
| 0 | FETCH | = branch instruction address | pc_enable=1 → PC increments to PC+1 on posedge; ir_load=1 → IR captures branch instruction |
| 1 | DECODE | = PC+1 (already incremented) | Combinational decode; no state changes |
| 2 | EXECUTE | = PC+1 | ALU executes SUB(Rd,Rs); flags Z,N,V,C updated |
| 3 | WRITEBACK | = PC+1 | Branch condition evaluated; if true: pc_load=1, PC ← PC+1 + sign_ext(imm6) |
| 4 | FETCH | = branch target (if taken) | Next instruction fetched from branch target |

## 4.2  Branch Target Formula

sign_ext_16 = {{10{imm6[5]}}, imm6}    // 6→16 signed BitExtender

branch_target = pc_out + sign_ext_16   // pc_out is already PC+1

Effective: target = (original_branch_PC + 1) + sign_ext(imm6)

target = original_branch_PC + 1 + offset

## 4.3  Signed Offset Range

| imm6 (6-bit two's complement) | Decimal | Jump destination | Notes |
| --- | --- | --- | --- |
| 011111 | +31 | PC + 32 | Maximum forward branch |
| 000001 | +1 | PC + 2 | Skip next instruction |
| 000000 | 0 | PC + 1 | Effectively NOP branch |
| 111111 | −1 | PC + 0 | Back to branch itself (infinite loop) |
| 111110 | −2 | PC − 1 | One instruction before branch |
| 111101 | −3 | PC − 2 | Two instructions before branch |
| 100000 | −32 | PC − 31 | Maximum backward branch |

The assembler (assembler.py) automatically computes offset = symbol_address − (PC + 1) when a label is used as a branch target, so assembly programs use labels rather than raw numeric offsets.

## 4.4  JMP — Absolute Address

jmp_target = {10'b0, imm6}    // Splitter zero-extends 6→16

JMP target range: 0 to 63 (addresses 0x00–0x3F only)

JMP uses the Splitter {6,10→16} path, not the BitExtender path. The upper 10 bits are Const(0,10-bit). This is why JMP is limited to the first 64 instructions in v0.3.

## 4.5  BLT — Signed Less-Than Condition

BLT uses the standard RISC signed comparison: N ⊕ V = 1 means the signed result of Rd−Rs is negative.

| Case | a[7] | b[7] | result[7] | N | V | N⊕V | Correct a<b? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 − 9 = −5 | 0 | 0 | 1 | 1 | 0 | 1 | Yes ✓ |
| 9 − 4 = +5 | 0 | 0 | 0 | 0 | 0 | 0 | No ✓ |
| −1 − 0 = −1 | 1 | 0 | 1 | 1 | 0 | 1 | Yes ✓ |
| 127 − (−128) overflows | 0 | 1 | 1 | 1 | 1 | 0 | No ✓ (overflow flips sign) |
| −128 − 127 overflows | 1 | 0 | 0 | 0 | 1 | 1 | Yes ✓ (overflow flips sign) |

In alu.dig this is implemented as: XOR(negative_flag, overflow_flag) → AND with is_WB and is_BLT in control_unit.dig → pc_load.

# 5.  Memory Architecture

## 5.1  Harvard Separation

Instruction memory and data memory are completely separate circuits with no shared buses. This eliminates structural hazards between FETCH (reading instructions) and LOAD/STORE (reading/writing data), which is the primary advantage of the Harvard model for future pipelining.

| Property | Instruction Memory (instruction_memory.dig) | Data Memory (data_memory.dig) |
| --- | --- | --- |
| Digital element | ROM | RAMDualPort |
| Depth | 256 × 16-bit words | 256 × 8-bit bytes |
| Address width | 8-bit (PC[7:0]) | 8-bit ({2'b00, imm6}) |
| Read mode | Asynchronous | Synchronous (registered) |
| Write | Not supported | Synchronous on posedge clk |
| Write enable | — | mem_write (Port A) |
| Read enable | Always on | mem_read (Port B) |
| Address source | program_counter output | imm6 zero-extended |

## 5.2  LOAD Timing

Because data_memory.dig uses RAMDualPort with a registered (synchronous) read output, LOAD requires two clock cycles to complete:

- EXECUTE: mem_read asserted; RAMDualPort Port B captures address on rising edge

- WRITEBACK: mem_read still asserted (held by control unit); read_data valid; reg_write=1 routes mem_data through the writeback MUX to the register file

The writeback MUX in cpu_core_test.dig has sel=mem_read. This means during any WRITEBACK cycle where mem_read is high, mem_data is selected over alu_result. Only LOAD assertions this combination.

## 5.3  Address Constraints (v0.3)

- Instruction memory: only the lower 8 bits of the 16-bit PC are used. Effective program size limited to 256 instructions.

- Data memory: address = {2'b00, imm6} = {0, 0, imm6[5:0]}. Accessible addresses are 0x00–0x3F (0–63). The upper 194 bytes of the 256-byte RAM are inaccessible in v0.3.

# 6.  Example Program — Cycle-by-Cycle Walkthrough

The program stored in instruction_memory.dig (and in example.asm) exercises all implemented instructions. Initial state: R0–R7 = 0, PC = 0, all flags = 0.

## 6.1  Program Listing

| Addr | Hex | Mnemonic | Effect |
| --- | --- | --- | --- |
| 0x00 | 0x9205 | ADDI R1, #5 | R1 ← 0 + 5 = 5 |
| 0x01 | 0x9405 | ADDI R2, #5 | R2 ← 0 + 5 = 5 |
| 0x02 | 0xB281 | BEQ R1, R2, +1 | R1==R2 → Z=1 → taken; PC ← 3+1 = 4 (skips 0x03) |
| 0x03 | 0x9601 | ADDI R3, #1 | SKIPPED — never fetched |
| 0x04 | 0x9809 | ADDI R4, #9 | R4 ← 0 + 9 = 9 |
| 0x05 | 0x780A | STORE R4, #10 | MEM[10] ← R4 = 9 |
| 0x06 | 0x6A0A | LOAD R5, #10 | R5 ← MEM[10] = 9 |
| 0x07 | 0x9C05 | ADDI R6, #5 | R6 ← 0 + 5 = 5 (loop counter init) |
| 0x08 | 0x9C3F | ADDI R6, #-1 | R6 ← R6 − 1 (LOOP top) |
| 0x09 | 0xDC01 | BLT R6, R0, +1 | if R6 < 0: PC ← 10+1=11 (exit) |
| 0x0A | 0xB03D | BEQ R0, R0, -3 | R0==R0 always → Z=1 → PC ← 11+(−3)=8 (back to LOOP) |
| 0x0B | 0x0000 | NOP | END sentinel — simulation terminates |

## 6.2  BEQ at 0x02 — Detailed 4-Cycle Trace

| Cycle | State | pc_out | Key signal values |
| --- | --- | --- | --- |
| 0 | FETCH | 0x02 | pc_enable=1 → PC increments to 0x03 at posedge; ir_load=1 → IR ← 0xB281 |
| 1 | DECODE | 0x03 | opcode=0xB(BEQ), rd=1, rs=2, imm6=000001=+1 (combinational) |
| 2 | EXECUTE | 0x03 | alu_op=0x2(SUB), a=R1=5, b=R2=5; result=0; Z=1, N=0, V=0, C=0 |
| 3 | WRITEBACK | 0x03 | is_BEQ & Z=1 → pc_load=1; branch_target=0x03+0x0001=0x04; PC ← 0x04 |
| 4 | FETCH | 0x04 | Instruction at 0x03 is never fetched |

## 6.3  BLT Exit — Final Loop Iteration

After 6 ADDI R6,#-1 operations (R6: 5→4→3→2→1→0→-1=0xFF):

- ADDI R6,#-1 with R6=0: imm6=111111, sign_ext_8=0xFF=−1; result=0x00+0xFF=0xFF; N=1, Z=0, C=0, V=0

- BLT R6,R0: SUB(0xFF, 0x00)=0xFF; N=1, V=0; N⊕V=1 → branch TAKEN

- branch_target = pc_out + sign_ext_16(+1) = 0x0A + 0x0001 = 0x0B (NOP)

Simulation output confirms: R6=255 (0xFF) in the final state, R1=5, R2=5, R4=9, R5=9, PC=12.

# 7.  Verilog ↔ Digital Circuit Correspondence

The Verilog files in src/ are a direct RTL translation of the .dig schematics. This section documents every structural decision made to match the circuit exactly.

| src/ file | Matches .dig | Key structural differences vs original Verilog |
| --- | --- | --- |
| decoder.v | decoder.dig | No change needed — was already a perfect Splitter match |
| program_counter.v | program_counter.dig | Rewrote to expose 3-MUX chain: after_enable → after_load → d_next. Previously used if/else priority chain. |
| instruction_register.v | instruction_register.dig | Rewrote: OR gate for enable, MUX for data select. Register only clocks when OR(reset,ir_load)=1. Previously always-enabled. |
| instruction_memory.v | instruction_memory.dig | Fixed memory[11]: was 0xA00B (JMP infinite loop), corrected to 0x0000 (NOP) |
| data_memory.v | data_memory.dig | Modelled RAMDualPort with same address bus to both ports; separate mem_read/mem_write enables. Previously single always block. |
| reg_file.v | reg_file.dig | Added generate loop for 7 AND-gate-gated registers (R1–R7). R0 stays initialised to 0 and never written. Read MUXes return Const(0) for read_reg=0. |
| alu.v | alu.dig | All 5 operations run in parallel. Carry MUX and overflow gate tree match .dig exactly (XNOR/XOR/AND + 1-bit MUX). |
| control_unit.v | control_unit.dig | Removed alu_op_latched and alu_src_latched registers — .dig has no such latches. Both are purely combinational. State counter uses mux-then-register topology. |
| cpu_core.v | cpu_core_test.dig | Confirmed BitExtenders are signed. JMP uses {10'b0,imm6}. is_JMP comparator matches Const(10). No alu_op/alu_src latches in ctrl instantiation. |
| fetch_unit.v | (no direct .dig file) | Convenience wrapper grouping PC, IMEM, IR as in cpu_core_test.dig topology |

# 8.  Build and Simulation Guide

## 8.1  Makefile Targets

| Target | Command | What it does |
| --- | --- | --- |
| sim | make sim | Compiles all src/*.v + tb/cpu_core_tb.v with iverilog, runs simulation, prints per-instruction register state |
| wave | make wave | Opens build/cpu_dump.vcd in GTKWave |
| asm | make asm | Assembles example.asm → build/example.mem + .hex + _init.v |
| verilator | make verilator | Builds and runs C++ simulation via Verilator using cpu_core_debug.v |
| clean | make clean | Removes build/ and obj_dir/ |

## 8.2  GTKWave — Recommended Signal List

Add these signals in order after opening build/cpu_dump.vcd:

| Signal path | Width | What to look for |
| --- | --- | --- |
| cpu_core_tb.uut.ctrl.state | 2-bit | FSM state: 00=FETCH, 01=DECODE, 10=EXECUTE, 11=WRITEBACK |
| cpu_core_tb.uut.pc_out | 16-bit | PC value; jumps on taken branches |
| cpu_core_tb.uut.opcode | 4-bit | Current instruction opcode |
| cpu_core_tb.uut.rd | 3-bit | Destination register index |
| cpu_core_tb.uut.rs | 3-bit | Source register index |
| cpu_core_tb.uut.imm6 | 6-bit | Immediate field |
| cpu_core_tb.uut.ctrl.ir_load | 1-bit | High in FETCH only |
| cpu_core_tb.uut.ctrl.pc_enable | 1-bit | High in FETCH only |
| cpu_core_tb.uut.ctrl.reg_write | 1-bit | High in WRITEBACK for arith/MOV/ADDI/LOAD |
| cpu_core_tb.uut.ctrl.alu_op | 4-bit | 0x2 for branches; opcode value for arithmetic |
| cpu_core_tb.uut.ctrl.alu_src | 1-bit | High for ADDI (selects imm over reg) |
| cpu_core_tb.uut.ctrl.mem_read | 1-bit | High in EX+WB for LOAD |
| cpu_core_tb.uut.ctrl.mem_write | 1-bit | High in WB for STORE only |
| cpu_core_tb.uut.ctrl.pc_load | 1-bit | High in WB when branch taken or JMP |
| cpu_core_tb.uut.zero | 1-bit | Z flag |
| cpu_core_tb.uut.negative | 1-bit | N flag |
| cpu_core_tb.uut.overflow | 1-bit | V flag |
| cpu_core_tb.uut.carry | 1-bit | C flag |
| cpu_core_tb.uut.alu_unit.result | 8-bit | Raw ALU output |
| cpu_core_tb.uut.rf.registers[1] | 8-bit | R1 |
| cpu_core_tb.uut.rf.registers[2] | 8-bit | R2 |
| cpu_core_tb.uut.rf.registers[4] | 8-bit | R4 |
| cpu_core_tb.uut.rf.registers[5] | 8-bit | R5 |
| cpu_core_tb.uut.rf.registers[6] | 8-bit | R6 (loop counter — changes each iteration) |
| cpu_core_tb.uut.dm.memory[10] | 8-bit | Data memory address 10 (STORE/LOAD target) |
| cpu_core_tb.uut.branch_target | 16-bit | Computed branch destination |

## 8.3  Verification Checkpoints

The following events should be visible in GTKWave at the stated times (10 ns clock period, 40 ns per instruction):

| Time (ns) | Expected observation |
| --- | --- |
| 45 | t=45: WRITEBACK of ADDI R1,#5 — R1 changes from 0 to 5 |
| 85 | t=85: WRITEBACK of ADDI R2,#5 — R2 changes from 0 to 5 |
| 125 | t=125: WRITEBACK of BEQ R1,R2,+1 — pc_load=1, zero=1, pc_out=3, next FETCH shows pc_out=4 |
| 165 | t=165: WRITEBACK of ADDI R4,#9 (pc_out=5, address 3 was skipped) |
| 205 | t=205: WRITEBACK of STORE R4,#10 — mem_write=1, dm.memory[10] becomes 9 |
| 245 | t=245: WRITEBACK of LOAD R5,#10 — mem_read=1, reg_write=1, R5 becomes 9 |
| 965 | t=965: Final BLT WRITEBACK — R6=255 (0xFF = −1 signed), pc_load=1, exits loop |
| 1005 | t=1005: NOP WRITEBACK — simulation terminates |

# 9.  Known Issues and Planned Improvements

## 9.1  Known Issues in .dig Files

| File | Issue | Severity | Fix |
| --- | --- | --- | --- |
| instruction_memory.dig | ROM entry [11] = 0xA00B (JMP #11) — creates infinite loop instead of halting | Critical | Change to 0x0000 (NOP) to match assembler output |
| data_memory.dig | RAMDualPort port mapping needs verification in Digital: same address wire must connect to BOTH the read-address and write-address inputs | Medium | Verify both ports share the single address In port in the .dig schematic |
| cpu_core_test.dig | Both BitExtender elements must be configured as signed (arithmetic) extension in Digital. Default is zero-extension; a wrong setting silently breaks negative immediates and backward branches | Critical | Open each BitExtender in Digital, set extension type to signed |
| control_unit.dig | alu_op chain of 5 MUXes should be verified in Digital to confirm correct sel signal routing, as the x-coordinate-based wire tracing is complex | Low | Trace and annotate each MUX sel signal in Digital |

## 9.2  Planned Features (v1.0)

- Register-indirect addressing for LOAD/STORE — removes the 0–63 data address constraint

- Wider JMP target — 12-bit immediate or register-indirect to reach beyond address 63

- Debug bus (cpu_core_debug.v already implemented) — connect to Verilator C++ wrapper or FPGA JTAG

- BGE, BGT, BLE branch variants

- Assembler directives: .data, .text, .equ

## 9.3  Not Implemented (Reserved for Future)

- PUSH, POP, CALL, RET — requires stack pointer (SP) integration

- Multiply / divide — require multi-cycle execution or dedicated hardware

- Shift / rotate operations

- Interrupt / exception handling

- Pipeline stages — planned post-v1.0; Harvard architecture already eliminates structural hazards

## 9.4  Pipelining Roadmap

When transitioning to a 4-stage pipeline (IF → ID → EX → WB):

| Hazard type | Example | Required mitigation |
| --- | --- | --- |
| RAW (data) | ADDI R1,#5 followed immediately by ADD R2,R1 | Forwarding unit or load-use stall insertion |
| Control (branch) | BEQ followed by instruction in branch shadow | Flush shadow instruction or branch predictor |
| Structural (mem) | Simultaneous LOAD fetch and data read | Already solved — Harvard model separates IMEM/DMEM |

# 10.  Appendix

## A.  Register Set

| Register | Width | Purpose | Special behaviour |
| --- | --- | --- | --- |
| R0 | 8-bit | General-purpose | Hardwired zero — writes silently discarded; reads always return 0x00 |
| R1–R7 | 8-bit | General-purpose | Normal read/write; no aliases |
| PC | 16-bit | Program Counter | Synchronous: reset→0, load→pc_in, enable→PC+1, else hold |
| IR | 16-bit | Instruction Register | Clocked only when ir_load=1 (FETCH) or reset=1 |
| FLAGS | 4-bit (Z,C,N,V) | ALU condition codes | Set by ALU combinationally; read by control_unit in WRITEBACK |
| MAR | 16-bit | Memory Address Register | Reserved — not wired in v0.3 |
| MDR | 8-bit | Memory Data Register | Reserved — not wired in v0.3 |
| SP | 16-bit | Stack Pointer | Reserved — not implemented in v0.3 |

## B.  Signed Offset Encoding Quick Reference

| Decimal offset | 6-bit two's complement | Where it branches (relative to branch PC) |
| --- | --- | --- |
| +31 | 011111 | PC + 32 (max forward) |
| +2 | 000010 | PC + 3 |
| +1 | 000001 | PC + 2 (skip next instruction) |
| 0 | 000000 | PC + 1 (no-op branch) |
| −1 | 111111 | PC + 0 (branch to itself — loop) |
| −2 | 111110 | PC − 1 |
| −3 | 111101 | PC − 2 (used by BEQ R0,R0,LOOP in example.asm) |
| −32 | 100000 | PC − 31 (max backward) |

## C.  Assembler — Format Reference

| Format | Mnemonic | Syntax | Example |
| --- | --- | --- | --- |
| FMT_NONE | NOP | NOP | NOP |
| FMT_RR | ADD SUB AND OR XOR MOV | MNEM Rd, Rs | ADD R1, R2 |
| FMT_RI | ADDI | ADDI Rd, #imm | ADDI R6, #-1 |
| FMT_MEM | LOAD STORE | MNEM Rd, #addr | STORE R4, #10 |
| FMT_BRANCH | BEQ BNE BLT | MNEM Rd, Rs, label | BEQ R1, R2, SKIP |
| FMT_JUMP | JMP | JMP label | JMP START |

Comments use semicolons (;). Labels end with a colon. Immediates are prefixed with #. Registers are R0–R7 (case-insensitive).

## D.  File Map

| File / directory | Purpose |
| --- | --- |
| cpu_digital/alu.dig | ALU circuit — parallel operation units + result MUX + flag logic |
| cpu_digital/control_unit.dig | Control unit — FSM counter + opcode comparators + control signal AND/OR trees |
| cpu_digital/cpu_core_test.dig | Top-level integration schematic — all components wired together |
| cpu_digital/data_memory.dig | Data RAM — RAMDualPort 256×8-bit |
| cpu_digital/decoder.dig | Instruction decoder — single Splitter |
| cpu_digital/instruction_memory.dig | Instruction ROM — 256×16-bit with example program |
| cpu_digital/instruction_register.dig | IR latch — MUX + OR gate + Register |
| cpu_digital/program_counter.dig | PC — Adder + 3×MUX + Register |
| cpu_digital/reg_file.dig | Register file — Decoder + 7×AND + 8×Register + 2×MUX8 |
| src/alu.v | RTL translation of alu.dig |
| src/control_unit.v | RTL translation of control_unit.dig (no alu_op latch) |
| src/cpu_core.v | RTL translation of cpu_core_test.dig |
| src/cpu_core_debug.v | cpu_core.v + debug bus outputs (v0.4 preview) |
| src/data_memory.v | RTL translation of data_memory.dig (RAMDualPort) |
| src/decoder.v | RTL translation of decoder.dig |
| src/fetch_unit.v | Convenience wrapper grouping PC + IMEM + IR |
| src/instruction_memory.v | RTL translation of instruction_memory.dig |
| src/instruction_register.v | RTL translation of instruction_register.dig |
| src/program_counter.v | RTL translation of program_counter.dig |
| src/reg_file.v | RTL translation of reg_file.dig |
| tb/cpu_core_tb.v | System-level testbench; terminates on NOP sentinel at PC≥12 |
| assembler.py | Two-pass Python assembler; outputs .mem, .hex, or Verilog init |
| example.asm | Demo program exercising all implemented instructions |
| Makefile | sim / wave / asm / verilator / clean targets |

Custom 8-bit RISC CPU  ·  Digital Circuit Design Reference  ·  v0.3